| Symbol | Terminal name | Symbol | Terminal Name |
| --- | --- | --- | --- |
| E01 | POWER GROUND | G- | DISTRIBUTOR |
| E02 | POWER GROUND | T | CHECK CONNECTOR |
| No. 1 | INJECTOR | OX1 | CHECK CONNECTOR |
| No. 2 | INJECTOR | OX2 | CHECK CONNECTOR |
| No. 3 | INJECTOR | KNK | KNOCK CONTROL SENSOR |
| No. 4 | INJECTOR | PIM | TURBO CHARGING PRESSURE SENSOR |
| STJ | COLD START INJECTOR | THW | WATER TEMP SENSOR |
| EGR | EGR VSV | IDL | THROTTLE POSITION SENSOR |
| RSC | ISC VALVE | THA1 | AIR TEMP SENSOR |
| RSO | ISC VALVE | VTA | THROTTLE POSITION SENSOR |
| HT | OXYGEN SENSOR HEATER | VS | AIR FLOW METER |
| IGT | IGNITER | VC | SENSOR POWER SOURCE |
| TPC1 | TURBO CHARGING PRESSURE VSV | E2 | SENSOR GROUND |
| TVIS | INTAKE AIR VSV | STA | STARTER SWITCH |
| FC | CIRCUIT OPENING RELAY | AC1 | A/C MAGNET SWITCH |
| IGF | IGNITER | ACT | A/C AMPLIFIER |
| G2 | DISTRIBUTOR | SPD | SPEED SENSOR |
| G1 | DISTRIBUTOR | FPR | FUEL PUMP RELAY |
| NE | DISTRIBUTOR | W | WARNING LIGHT |
| E1 | ENGINE GROUND | STP | STOP LIGHT SWITCH |
| +B1 | MAIN RELAY | ELS | TAILLIGHT AND DEFOGGER |
| BATT | BATTERY | +B | MAIN RELAY |
| VF | CHECK CONNECTOR |  |  |

Left Connector

| Top | E01 | No. 1 | No. 2 | RSO | RSC | HT | STJ | EGR | G2 | NE | IGF | TPC | TVIS |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Bottom | E02 | No. 3 | No. 4 | EVP |  | NEO | IGT |  | G1 | G- |  | RLY | E1 |

Middle Connector

| Top | VF | VTA2 | OX | PIM | THW | THA | VS | VC |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Bottom | IDL2 | TE1 | TE2 | KNK | IDL1 | VTA1 | THG | E2 |

Right Connector

| Top | STA | AC | SPD | PS | PSCT | FPR | W | STP | EGW | ELS | BATT |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Bottom | ECO | ACT | TR1 | TR2 | VTO1 | VTO2 | CC0 | ABS | FC | +B1 | +B |

| Top | ID | E01 | No. 1 | No. 2 | RSO | RSC | HT | STJ | EGR | G2 | NE | IGF | TPC | TVIS |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Main Colour | White | Red | Green | Green | Green | Pink | Green | Green | Blue | Yellow | Black | Blue | Pale Green |
|  | Stripe Colour | Black |  |  | Black | White | Black | Red | Red |  |  | Yellow | Red |  |
|  | Band Colour |  |  |  |  |  |  |  |  |  |  | Red |  |  |
| Bottom | ID | E02 | No. 3 | No. 4 | EVP |  | NEO | IGT |  | G1 | G- |  | RLY | E1 |
|  | Main Colour | White | Yellow | Blue | Pink | NO WIRE | NO WIRE | White | NO WIRE | Red | Black | NO WIRE | Blue | Brown |
|  | Stripe Colour | Black |  |  |  |  |  |  |  |  |  |  | Yellow |  |
|  | Band Colour |  |  |  | Red |  |  | Red |  |  |  |  | Red |  |

| Top | ID | VF | VTA2 | OX | PIM | THW | THA | VS | VC |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Main Colour | Purple | NO WIRE | White | Red | Red | Grey | Grey | Pink |
|  | Stripe Colour | Yellow |  |  | White |  |  | Black | Blue |
|  | Band Colour | Red |  |  | Blue | Blue | Red | Red | Red |
| Bottom | ID | IDL2 | TE1 | TE2 | KNK | IDL1 | VTA1 | THG | E2 |
|  | Main Colour | NO WIRE | Orange | Pale Green | Translucent | Pink | Pink | NO WIRE | Brown |
|  | Stripe Colour |  |  |  |  |  | Black |  |  |
|  | Band Colour |  | Red | Red |  |  |  |  | Blue |

| Top | ID | STA | AC | SPD | PS | PSCT | FPR | W | STP | EGW | ELS | BATT |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Main Colour | Black | Black | Blue | NO WIRE | Yellow | Green | Pink | Green | NO WIRE | Orange | Pink |
|  | Stripe Colour |  | White | White |  |  | Red |  | White |  |  |  |
|  | Band Colour | Silver | Silver | Silver |  | Silver | Silver | Silver | Silver |  | Silver | Silver |
| Bottom | ID | ECO | ACT | TR1 | TR2 | VTO1 | VTO2 | CCO | ABS | FC | +B1 | +B |
|  | Main Colour | NO WIRE | Pale Green | NO WIRE | NO WIRE | NO WIRE | NO WIRE | NO WIRE | Pink | Green | Black | Black |
|  | Stripe Colour |  | Black |  |  |  |  |  | Black |  | Yellow | Yellow |
|  | Band Colour |  | Silver |  |  |  |  |  | Silver | Silver | Silver |  |